****************************************************************

LIBERO AND SOFTCONSOLE VERSIONS

****************************************************************

This application is tested with following

Libero and SoftCosole versions.

Libero SoC   --  Version 10.1

SoftConsole  --  Version 3.3

******************************************

******************************************

DESIGN FILE DIRECTORY STRUCTURE

******************************************

This ReadMe file explains the directory structure in the A2F_AC362_DF.zip package. After unzipping the package you will find the below directory structure:

A2F_500 contains two separate directories for programming FPGA Fabric (Programming_FPGAFabric) and programming eNVM (Programming_eNVM).  A2F_200 contains solutions for A2F200 evaluation board. The following procedure applies for evaluation board also though it talks about development board.

Note:

When you are running the design on A2F200 evaluation kit board, use files from the path C:\A2F_AC362_DF\A2F_200.

When you are running the design on A2F500 development kit board, use files from the path C:\A2F_AC362_DF\A2F_500.

If A2F500 development kit has OLEDs, use oled_driver folder to find oled driver files and *.dat file to run the design.

When you regenerate the MSS component from Libero SoC project, ensure the following lines commented out in softconsole  project  workspace .

hw_platform library module:

File name : DirectC/dpcom.c

Line number 125: //#error "Please add code here to get the programming data. Please refer to the Required Source Code Modifications section of the DirectC user's guide."

File name : DirectC/dpuser.c

Line number 15 : //#include "main.h"

Line number 48 ://#error "Please modify this function to time delays. Please refer to the Required Source Code Modifications section of the DirectC user's guide."

Programming_eNVM:

In all these solutions, first you need to program the SmartFusion Hardware with the intended design configuration before using SoftConsole project to program the eNVM. The A2F_500\Programming_eNVM directory contains the required Libero design files to program the SmartFusion development kit board. The Libero design files available in A2F_500\Programming_eNVM \SmartFusion_IAP_HW  for both Solution 1 and Solution 3.

After programming the hardware design files with the intended configuration use the SoftConsole projects and host to program the eNVM. The A2F_500\Programming_eNVM\SmartFusion_IAP_HW\SoftConsole\MSS_IAP_MSS_CM3_0\projects directory contains two directories  IAP_with_UART_SPIFLASH and UART_HostPC  to program eNVM either from the onboard storage (SPI FLASH) or directly from the host PC.

IAP_with_UART_SPIFLASH directory represents Solution 3, Programming eNVM from the On-Board SPI Flash. To program eNVM, first load the SPI Flash with the eNVM programming file (.dat) and then program eNVM using IAP interface. IAP_with_UART_SPIFLASH directory contains UART_SPIFlashLoad_prj, IAP_Program_prj and a host_tool. Use host_tool and SoftConsole project UART_SPIFlashLoad_prj to program SPI Flash with eNVM programming file (.dat). Once the SPI Flash is programmed you can use IAP_Program_prj to program eNVM using IAP interface.

- For softconsole work space ,double-click Write Application Code underDevelop Firmware in the Libero SoC design flow window.Host tool is available at A2F_AC362_DF\A2F_500\Programming_eNVM\SmartFusion_IAP_HW\SoftConsole\MSS_IAP_MSS_CM3_0\IAP_with_UART_SPIFLASH\host_tool

UART_HostPC directory represents Solution 1, Programming eNVM Directly from the Host PC using UART. This directory contains the SoftConsole project  IAP_UART_From_HostPC and a host_tool. Use host_tool and SoftConsole project IAP_UART_From_HostPC to get the eNVM programming file(.dat) from the host PC into internal memory on the target board and program IAP interface.

- For softconsole work space ,double-click Write Application Code underDevelop Firmware in the Libero SoC design flow window.Host tool is available at \A2F_AC362_DF\A2F_500\Programming_eNVM\SmartFusion_IAP_HW\SoftConsole\MSS_IAP_MSS_CM3_0\UART_HostPC\host_tool

For both Solution 1 and Solution 3 the eNVM programming file (*.dat) is available in \A2F_AC362_DF\A2F_500\Programming_eNVM\LED. You need to provide this .dat file when you run Memory loader on desktop. For A2F200 evaluation board, \A2F_AC362_DF\A2F_200\Programming_eNVM\eNVM

Programming_FPGA Fabric:

In all the solutions, first you need to program the SmartFusion development kit board with the intended design configuration before using SoftConsole project to program the FPGA fabric.

The A2F_AC362_DF \A2F_500 directory contains the required Libero design files to program the SmartFusion development kit board. For Solution 1, Solution 3 use the Libero design files available in SmartFusion_IAP. These Libero design files are available in \A2F_AC362_DF\A2F_500\Programming_FPGAFabric\SmartFusion_IAP.

After programming the hardware design files with the intended configuration, invoke  the SoftConsole, double-click Write Application Code underDevelop Firmware in the Libero SoC design flow window to program FPGA fabric.

The A2F_AC362_DF\A2F_500\Programming_FPGAFabric\SmartFusion_IAP\SoftConsole\MSS_IAP_MSS_CM3_0\projects directory contains two main directories SPI_FLASH and UART_HostPC.

SPI_FLASH directory represents Solution 3, Programming FPGA Fabric from the On-Board SPI Flash. To program FPGA fabric, first load the SPI Flash with the FPGA fabric programming file (.dat) and then program FPGA fabric using IAP interface. SPI Flash can be programmed either using UART or Ethernet with TFTP. This directory contains work spaces IAP_with_Ethernet_SPIFLASH and IAP_with_UART_SPIFLASH for programming SPI Flash in both UART and Ethernet with TFTP methods. In the method Ethernet with TFTP, select Ethernet_SPIFlashLoad in softconsole workspace for loading the FPGA fabric programming file (.dat) into SPI Flash and select IAP_Ethernet_prj to program FPGA fabric using IAP interface.Please make sure TFTP is enabled in your host PC before proceeding. In windows 7 operating system , you can enable TFTP under control panel->programs->turn Windows features on or off . In UART method, select UART_SPIFlashLoad in softconsole workspace for loading the FPGA fabric programming file (.dat) into SPI Flash and select IAP_UART_prj to program FPGA fabric using IAP interface .

- Host tool is available for UART method in

A2F_AC362_DF\A2F_500\Programming_FPGAFabric\SmartFusion_IAP\SoftConsole\MSS_IAP_MSS_CM3_0\SPI_FLASH\IAP_with_UART_SPIFLASH\host_tool

UART_HostPC directory represents Soultion 1, Programming FPGA Fabric Directly from the Host PC using UART. In this method, select IAP_UART_From_HostPC in softconsole workspace and Use the host_tool    to load the  FPGA fabric  programming file from the host PC into internal memory on the target board and program FPGA fabric using IAP interface.

- host tool for programming FPGA fabric directly from the host PC using UART method is available in \A2F_AC362_DF\A2F_500\Programming_FPGAFabric\SmartFusion_IAP\SoftConsole\MSS_IAP_MSS_CM3_0\UART_HostPC\host_tool

For Solution 2 use the Libero design files available at \A2F_AC362_DF\A2F_500\Programming_FPGAFabric\Ethernet_Host_PC_IAP.

After programming the hardware design files with the intended configuration, invoke  the SoftConsole, double-click Write Application Code underDevelop Firmware in the Libero SoC design flow window to program FPGA fabric.

The \A2F_AC362_DF\A2F_500\Programming_FPGAFabric\Ethernet_Host_PC_IAP\SoftConsole\SmartFusion_Ethernet_IAP_MSS_MSS_CM3_0\projects directory contains mainly 2 directories Ethernet_From_HostPC_IAP and hosttool.

For Solution 2, select Ethernet_From_HostPC_IAP project under softconsole for   Programming FPGA Fabric Directly from the Host PC using Ethernet.

host_tool is provided at

\A2F_AC362_DF\A2F_500\Programming_FPGAFabric\Ethernet_Host_PC_IAP\SoftConsole\SmartFusion_Ethernet_IAP_MSS_MSS_CM3_0\hostTool

The FPGA fabric programming files (.dat) for different LED sequence are available under below directory structure.

- A2F_AC362_DF\A2F_500\Programming_FPGAFabric\LED_sequence_13_24

- A2F_AC362_DF\A2F_500\Programming_FPGAFabric\LED_sequence_12_34

- A2F_AC362_DF\A2F_500\Programming_FPGAFabric\LED_sequence_14_23

You need to provide this *.dat file when you run Memory loader on desktop.

For A2F200 evaluation board , You can find FPGA fabric programming files (.dat) for different LED at A2F_AC362_DF\A2F_200\Programming_FPGAFabric folder.

A2F200

============================

This folder consists of Libero SOC Verilog projects for A2F200  based SmartFusion Evaluation board.

These project files will not work with A2F500 based  SmartFusion Development board.

Also, the software is now a part of Libero SoC and can be found in the Softconsole folder.

A2F500

============================

This folder consists of Libero SOC Verilog for A2F500 and   based  Smart Fusion Development board.

These project files will not work with A2F200 based SmartFusion

Evaluation board and SmartFusion Development board.

Also, the software is now a part of Libero SoC and can be found in the Softconsole folder.

Programming File

****************

This folder consists of programming file (*.STP) for A2F200 based SmartFusion Evaluation board and A2F500 SmartFusion Development board.