## Adaptive FIR Filter Demo GUI

- The Adaptive FIR Filter Demo is provided with a user friendly GUI that runs on the host PC to communicate with the SmartFusion2 starter kit. UART is used as the communication protocol between the host PC and the SmartFusion2 starter kit.

Adaptive FIR Filter Demo GUI

- The GUI consists of the following   tabs:

- Input Parameters: Configures the serial COM port and Signal Generation

- Filter Output: Plots Error Signal and its frequency spectrum

- Text viewer: Shows Input Signal, Error Signal data values

Click Help for more information on the GUI as shown in Figure 2.

Adaptive FIR Filter Demo GUI Help

## Running the Design

- Program the SmartFusion2 device with the programming file provided in the design files (\\Adaptive_FIR_Filter_Demo\Programming File\Adaptive_FIR_filter_demo.stp) using the FlashPro design software.

- Launch the Adaptive FIR Filter Demo GUI executable file available in the design files. (\\Adaptive_FIR_Filter_Demo\GUI\SF2_Adaptive_FIR_Filter.exe). The GUI window is displayed, as shown in Figure 3.

Serial Port Configuration

- Serial Port Configuration: The COM port number is automatically detected and baud rate is fixed at 11, 5200. Press Connect as shown in Figure 4.

- Signal Generation:

- Enter the Narrowband Signal frequency as 2 MHz (range 1 MHz to 20 MHz), Click “Generate ” as shown in Figure 10.

Signal Generation

- GUI will add the Wide band signal (generated inside the GUI)to the Narrowband Signal component and plots the combined signal(Narrow band + Wide band) , FFT spectrum as shown in Figure 5.

Signal Generation

- By clicking on “Start” button as shown in Figure 6. GUI loads the input data     (1 k samples) to the SmartFusion2 device for processing the filtering operation.

Adaptive FIR Filter Demo Start

- After completing the filter operation, the GUI receives the error data and its FFT data from SmartFusio2 and plots as shown in Figure 7. Error Signal plot shows the suppression of narrow band component from wideband signal after suitable amount of iterations.

Error Signal: Time and Frequency plot

- The narrow band signal component is suppressed in the Error Signal frequency spectrum. This can be observed in Error Signal FFT plot as shown in Figure 8.

Error Signal: Time and Frequency plot

- Now press Compare button as shown in Figure 9.

Error Signal: Time and Frequency plot

- Input Wide band vs Output Wide band comparison window is pop up as shown in Figure 10.

Input wide band vs Output Wide band

- Zoom in as shown in Figure 11.

Input wide band vs Output Wide band

- Compare the Error Signal (output wideband signal),with input wide band signal as shown in Figure 12. It is observed that the narrow band interfering component is eliminated and the wide band signal is preserved in error signal.

Input wide band vs Output Wide band

- Click Close button as shown in Figure 13.

Input wide band vs Output Wide band

- Right click on the window. It shows the different options as shown in Figure 14. The data can be copied, saved and exported to CSV plot for analysis purpose. Page setup, print, show point values, Zoom, and set scale are set to default.

Error Signal – GUI options

- The Input Signal, Error Signal values can  be viewed in Text Viewer. Click on the Text Viewer tab and then click on the corresponding View button as shown in Figure 15.

Text Viewer

- The values can be observed as shown in Figure 16.

Text Viewer: Input signal Values

- To save the Input Signal as a text file, right-click on the Input Signal window. It shows the different options as shown in Figure 17.

- Click Save . Select OK to save the text file.

Text Viewer - Coefficients Save Options

- Click Exit to stop the demo as shown in Figure 18.

Exit  Demo