SPI Flash Loader and Code Shadowing Demo GUI is a simple graphic user interface to program the SPI Flash and running the code shadowing demo on the SmartFusion2 validation board.

# Figure 1 shows the Flash loader GUI.

The Flash loader consists of four sections:

- Serial port configuration

- File location

- Options

- Serial console

# Serial port configuration

It has the configurations COM port and Baud Rate to establish the connection with the MMUART_0 of the SmartFusion2. Default Baud Rate will be 115200. To select the COM port and baud rate use the drop down option provided beside the corresponding configuration.

# File location

It has an option to browse the application image to program the SPI Flash. The address field has the default address as 0x00000000. Use the default address. For the options code shadowing from DDR and code shadowing from SDR the File Browsing option automatically disabled.

# Options

GUI shows the below options. Select the option accordingly with the help of radio buttons.

Program: To load the image file into SPI flash.

Program and Code Shadowing from DDR: To load the image file into SPI flash and boot the same image from DDR memory

Program and Code Shadowing from SDR: To load the image file into SPI flash and boot the same image from SDR memory

Code Shadowing from DDR: To boot the existing image file in the SPI Flash from DDR memory.

Code Shadowing from SDR: To boot the existing image file in the SPI Flash from SDR memory.

# Serial console

A console window is provided to print the messages. It will not support to enter the data through the serial console. The serial console also has below options and the options can be selected by right clicking in the console.

# Select all: to select the complete text in the console

Copy: to copy the selected text

Clear: to clear the text on the console

Save: to save the text in the console to a location in the host PC.

# Warning Dialog boxes:

The GUI shows several warning dialog boxes on the occurrence of wrong configuration.

- If you programmed with a STAPL file in which MDDR is not configured for DDR memory and you selected option for code shadowing to DDR then it shows an error message as shown below.

or

If you programmed with a STAPL file in which MDDR is not configured for SDR Controller and you selected option for code shadowing to SDR memory then it shows an error message as shown below.

Wrong option

- If you click start without connecting COM port below warning appears

Warning

- If you click start to program SPI Flash without giving the image file, it shows below warning

Browse hex file