<Product Area>

<Device>

API Specification for

Linux Host Bus Interface Driver

Draft Version 0.1

April 2015

# Table of Contents

1	Introduction	5

1.2	Purpose of the Document	5

3.1	Scope	5

3.1	Abbreviations	5

3.1	References	5

3.1	Assumptions	5

2	Design Approach	5

3	Public Data Structure	6

3.1	IOCTL	6

3.2	HBI_LNX_DRV_FLASH_ERASE_ARGS	7

3.3	HBI_LNX_DRV_FLASH_LOAD_FWR_ARGS	7

3.4	HBI_LNX_DRV_RW_ARGS	7

3.5	Compile – Time Options	7

4	PROCFS Support	7

4.1	open_device	8

4.2	close_device	8

4.3	write_reg	9

4.4	read_reg	9

4.5	cfgrec	9

4.6	load_fw	9

4.7	start_fw	9

4.8	flash_erase	9

4.9	flash_save_fwrcfgrec	9

4.10	flash_load_fwrcfgrec	10

4.11	PROCFS Usage Examples	10

5	HBI Boot Option	12

List of Tables

Table 1 Abbreviations used in this document	5

List of Figures

No table of figures entries found.

| Document Owner | Shally Verma |
| --- | --- |
| Version | Draft |
| Date | 8 April 2015 |

| Document Change History |  |  |  |
| --- | --- | --- | --- |
| Date | Version | Changed by | Change Description |

# Introduction

This document describes the linux kernel driver for Host Bus Interface driver for Microsemi VPROC device family.

## Purpose of the Document

This document outlines the features exported and supported by HBI linux kernel driver

## Scope

This document is intended to be used by Applications using Microsemi VPROC device family on linux platform.  Currently it support only kernel level driver. User space is not covered.

## Abbreviations

| Table 1 Abbreviations used in this document |  |
| --- | --- |
| Abbreviation | Explanation |
| HBI | Host Bus Interface |
| VPROC | Voice Processor |

## References

[1] ZL38040/50/60/80/51 firmware Manual

[2] ZL38004/ZL38005/ZL38012 firmware Manual

[3] HBI_Driver_API_Specification.doc

## Assumptions

This document assumes that user is aware of HBI Driver for Microsemi VPROC device family.

# Design Approach

HBI linux driver is an abstraction layer on top of HBI for users on linux platform. This driver support various options to access HBI driver features from both user and kernel space.

During initialization, HBI linux driver primarily will do:

- Initialization of procfs interface to open, close, boot, configure and read or write access to VPROC device over HBI

- Registration as “character driver” to user island and accessible as /dev/hbi<major>:minor_num where Minor Num is fixed to 1.

- Support IOCTLs to open, close, boot, configure and read or write access to VPROC device

One single HBI driver instance would support multiple devices. Once driver is initialized, user can open any device in a system using HBI_OPEN ioctl call with HBI_DEV_CFG (containing device address information) as associated IOCTL argument OR proc interface ( for details, see PROCFS Support Section)

*We had an alternate option to register 1 driver instance per device. In that case, driver would have registered different character device node for each device to kernel. However, that approach required compile time mapping of device address to device numbers. To keep driver flexible to dynamically select and open device based on device actual address, we decided to open 1 driver instance and expose OPEN IOCTL call accepting device address and other configuration as IOCLT argument.

# Public Data Structure

Public header file for HBI linux kernel driver would primarily consists of IOCTL defines and associated arguments types.

## IOCTL

Below table summarize list of IOCTL as supported by HBI linux kernel driver along with expected input argument

| IOCTL | Description | Input | Output |
| --- | --- | --- | --- |
| HBI_OPEN | This IOCTL will open a requested device | HBI_DEV_CFG structure with bus number and device address information | Output of this call will be handle to opened device |
| HBI_CLOSE | This IOCLT will close a requested device | Device handle of the type unsigned int | None |
| HBI_WRITE | Write given buffer to requested device memory | HBI_LNX_DRV_RW_ARGS (see details below) | None |
| HBI_READ | Reads into a user buffer from device | HBI_LNX_DRV_RW_ARGS (see details below) | None |
| HBI_LOAD_FWR | Loads a boot image from host to device | HBI_DATA | None |
| HBI_START_FWR | Run the firmware loaded into RAM | None | None |
| HBI_FLASH_ERASE | Erases whole or specific image(if give) on flash. If null argument is passed, then IOCTL function would erase whole flash else retrieve specific image number from argument list and erase that. | None or HBI_LNX_DRV_FLASH_ERASE_ARGS. | None |
| HBI_FLASH_SAVE_FWR | Save Firmware and Configuration record in RAM to flash | None | Image number of image just written |
| HBI_FLASH_LOAD_FWR | Loads requested firmware and configuration record from RAM | HBI_LNX_DRV_FLASH_LOAD_FWR_ARGS |  |

## HBI_LNX_DRV_FLASH_ERASE_ARGS

typedef struct

{

HBI_STATUS status;

__u32  image_number; /* firmware image number to be erased from flash */

}HBI_LNX_DRV_FLASH_ERASE_ARGS;

## HBI_LNX_DRV_FLASH_LOAD_FWR_ARGS

Argument associated with HBI_FLASH_LOAD_FWR IOCTL

typedef enum struct hbi_lnx_drv_load_fwr_args{

HBI_STATUS status;

__u32    image_number; /* image number of firmware image on flash to read from */

} HBI_LNX_DRV_LOAD_FWR_ARGS;

## HBI_LNX_DRV_RW_ARGS

This data type is an input with HBI_READ and HBI_WRITE IOCTL.

typedef struct hbi_lnx_drv_rw_arg

{

HBI_HANDLE handle; /* handle as returned by call to HBI_OPEN IOCTL */

HBI_STATUS status; /* status of call */

void       *to;  /* destination location to read/write to */

void       *from; /* source location to read/write from */

size_t      len;  /* len of data in bytes(should be multiple of 2)*/

}HBI_LNX_DRV_RW_ARG;

## Compile – Time Options

Below table summarizes the different compile time options influencing driver behavior and feature set.

| Option | Description |
| --- | --- |
| HBI_ENABLE_PROCFS | If defined, then HBI driver will expose PROCFS interfaces |
| FLASH_PRESENT | Enable / Disable proc fs and IOCTL for flash operations support. |
| HBI_BUF_SIZE | Maximum buffer size used by HBI driver |

# PROCFS Support

HBI linux kernel driver initialize and create proc entries if compiled with HBI_ENABLE_PROCFS defined.

HBI linux kernel driver procfs interface looks something like this:

## open_device

This interface opens a VPROC device. It should be called before invoking any other interface.

For device at i2c bus, it takes bus number and device address as an input.

echo <bus_num>:<dev_addr(in hex)> > /proc/hbi/open_device

Example, to open device at i2c bus 1 with slave address 0x45 give

echo 1:45 > /proc/hbi/open_device

If device is opened successfully, then procfs entry will be created with name “dev_145” (where 1 is bus id and 45 device address in hex) for further operations on device.

## close_device

Write to this file closes the requested device. It takes device address (in hex) as input.

Example, to close device 0x45 enter

echo 45 > /proc/hbi/close_device

If device is closed successfully, then user should not see dev_<busnum,addr> entry

## write_reg

Write to this file writes data from user buffer to device register. It takes register address and data bytes in hex as input. Example, to write 0x12345678 at register 0x000C from device at address 0x45 on i2c bus 1

echo 000C 12345678 > /proc/hbi/dev_145/write_reg

## read_reg

Write to this file reads data from device memory to a user buffer. It takes register address and size as an input. Address in hex and size as number of bytes in decimal (should be multiple of 2).

Example to read 4 bytes from register 0x000c from device at address 0x45 on i2c bus 1

echo 000C 4 > /proc/hbi/dev_145/read_reg

if output of the command isn’t visible to console, run “dmesg” after command is complete

## cfgrec

Read of this file will display current configuration record in device.

Write to this file will update device at address 0x45 on bus 1 on i2c with user passed configuration record file. User can update configuration record with command

cat <cfgrec_file_name.cr2> > /proc/hbi/dev_145/cfgrec

## load_fw

Write to this file will load boot image into driver. Example, to load boot image for device at address 0x45 at i2c bus 1

cat <firmware image file name.s3> > /proc/hbi/dev_145/load_fw

## start_fw

Read to this file will write image to file loaded by load_fw interface. Example, to start downloading of boot image loaded on device with address 0x45 at i2c bus 1

cat /proc/hbi/dev_145/start_fw

This command should be executed immediate after load_fw or flash_load_fwrcfgrec command else it may return an error.

## flash_erase

Applicable if flash is present. Write to this file will erase inputted firmware image and associated configuration record from flash. Example to erase image 1 from device at address 0x45 on i2c bus 1

echo 1 > /proc/hbi/dev_145/flash_erase

Read to this file will simply erase whole flash

cat /proc/hbi/dev_145/flash_erase

## flash_save_fwrcfgrec

Read to this file will save current loaded firmware and configuration record in RAM on flash. This is supposed to be used immediate after start_fw command. Example to save firmware on flash at device 0x45 i2c bus 1

cat /proc/hbi/dev_145/flash_save_fwrcfgrec

## flash_load_fwrcfgrec

Write to this file will read given firmware image and associated configuration record from flash. Example, to read firmware image number 1 from flash into VPROC device ram at address 0x45 i2c bus 1 give

echo 1 > /proc/hbi/dev_145/flash_load_fwrcfgrec

## PROCFS Usage Examples

Prerequisite

Load hbi linux module:

insmod hbi.ko (if user have root permission) or

sudo insmod hbi.ko

User should see “hbi” directory created under /proc i.e.

/proc/hbi as in snapshot below:

    - Open device at address 0x45 i2c bus 1

    - Close device address 0x45 i2c bus 1 (remove entry dev_145)

    - Updating firmware on device and save it to flash

Open -> load_fw -> flash_save_fwrcfgrec

- echo 1:45 > /pro	c/hbi/open_fw

- cat <filename>.s3 > /proc/hbi/dev_145/load_fw

- cat /proc/hbi/dev_145/start_fw (download firmware to device)

- cat /proc/hbi/dev_145/flash_save_fwrcfgrec (save downloaded firmware to flash)

4.11.4	 Updating firmware on device and run it from RAM

- echo 1:45 > /proc/hbi/open_device

- cat <filename>.s3 > /proc/hbi/dev_145/load_fw

- cat /proc/hbi/dev_145/start_fw

- echo 1 > /proc/hbi/dev_145/start_fw

    - Boot device from specific image from flash.

- echo 1:45 > /proc/hbi/open_device (open device i2c bus 1 address 0x45)

- echo 2 > /proc/hbi/dev_145/flash_load_fwrcfgrec (read image number 2 from flash)

- echo 1 > /proc/hbi/dev_145/start_fw (set to start firmware execution)

# HBI Boot Option

HBI linux kernel driver support two interfaces for boot from host: procfs and ioctl. In operations, ioctl interface is similar to procfs interface only exception is ioctl command takes buffer (either precompiled or runtime read from a file by an application) as an input whereas procfs take file as an input.

  - PROCFS interface

HBI linux driver expose following device specific interfaces.

/proc/hbi/dev_<addr>/load_fw

/proc/hbi/dev_<addr>/start_fw

/proc/hbi/dev_<addr>/flash_save_fw

These interfaces should be used in following sequence

- Load firmware image from host and start its execution:

  - /proc/hbi/dev_<addr>/load_fw  /proc/hbi/dev_<addr>/start_fw

- Load firmware from host and save it to flash

  - /proc/hbi/dev_<addr>/load_fw->/proc/hbi/dev_<addr>/flash_save_fw

- Load firmware from host and save it to flash and execute it

  - /proc/hbi/dev_<addr>/load_fw -> /proc/hbi/dev_<addr>/flash_save_fw  /proc/hbi/dev_<addr>/start_fw

For usage for each procfs interface, refer to section 4 PROCFS Support

  - IOCTL interface

HBI_LOAD_FW, HBI_FLASH_SAVE_FW and HBI_START_FW are the supported ioctls for boot image loading. Every HBI_LOAD_FW call should be followed by HBI_START_FW and/or HBI_FLASH_SAVE_FWR.

Please refer to IOCTL section for more details.