# AWS IOT on SecureIOT1702

## Hardware

SecureIOT CEC1702 board – For running the AWS IOT demo application

Winc1510 Clicker board – connected on mikroBus 1. This provides the Wifi connectivity

## Tools/Utilities

Keil - For project build, downloading and debugging code

DediProg SF 100 – For loading spi image in flash

ComXDBG.exe – UART host application for viewing trace messages from secureIOT1702 board

## Build firmware

The codetree secureIOT1702_led_demo contains the project and source files for compiling and building the keil project.

### Project File

The Keil project file path:

secureIOT1702_led_demo\led_demo_project\aws_iot_cec1702.uvprojx

### mbedTLS

mbedTLS is added in the project as a library

secureIOT1702_led_demo\framework\aws\external_libs\mbedTLS\libmbedtls240.lib

### AWS IOT C SDK

AWS IOT C SDK path:

secureIOT1702_led_demo\framework\aws

The SDK is from AWS github - https://github.com/aws/aws-iot-device-sdk-embedded-C

The SDK is updated for CEC1702 platform.

The platform specific adapters for CEC1702:

- secureIOT1702_led_demo\framework\aws\platform\cec1702\common\timer.c

- secureIOT1702_led_demo\framework\ aws\platform\cec1702\mbedtls\ mbedtls_net.c

- secureIOT1702_led_demo\framework\ aws\platform\cec1702\ mbedtls\ network_mbedtls_wrapper.c

- secureIOT1702_led_demo\framework\ aws\platform\cec1702\ mbedtls\ winc1500_connect.c

- secureIOT1702_led_demo\framework\ aws\platform\cec1702\pthread\ threads_pthread_wrapper.c

Note: The demo application doesn’t have any RTOS or kernel, it is a simple loop; hence thread wrapper functions are empty stub functions.

### AWS Application

The AWS application is subscribe_publish_app. It uses MQTT as the transport for communicating to the AWS IOT cloud.

This file is same as one provided with AWS SDK samples; it has been updated for CEC1702 led demo.

The certificates and keys required for authentication using X.509 is hardcoded in the subscribe_publish_app.c file.

- Root Certificate  :  char rootCA[ ] =  “  “

- Client Certificate : char clientCRT[ ] =  “ “

- Client Private Key  : char clientKey[ ] ==  “ “

Configuration for AWS connectivity is done through aws_iot_config.h file.  Update the host, client_id and thing name based on your AWS setup.

### WiFi Configuration

Currently the code is configured to connect through WPA-PSK. The SSID and password are set statically in winc1500_connect.c

Modify the above values to match your wifi router.

### Compiler

- Keil uVision V5.20.0.0

### Output

The spi_image is created as part of the post-build process. The output files are placed in:

secureIOT1702_led_demo\led_demo_project\targets

### Code Tree Structure

## Build Firmware into Flash

Use DediProg SF 100 tool to program the image – spi_image.bin (created during the postbuild process) into flash on the secureIOT1702 board.

## ComXDBG

To view the UART traces:

- connect USB cable between secureIOT1702 board and Windows host

- After driver installation, start ComXDBG.exe

secureIOT1702_led_demo\utilities\ComEDBG\ComXDBG.exe

- Select FTDIBUS COM port

For example; for the below options we would enter 0

- You should be able to view UART traces from the secureIOT1702 board

### Sample log

[11:45:11.175] ORIGINAL IMAGE

[11:45:11.175] AWS IOT Node

[11:45:11.175] Microchip CEC1702

[11:45:11.175] HW Version: SECUREIOT1702_REV_B0

[11:45:11.175] FW Version/Label: SECUREIOT1702_AWS_LED_0100

[11:45:11.175] May 11 2017 11:43:05

[11:45:11.479] $Ts:

[11:45:12.033] winc1500_wifi_cb: 2c

[11:45:12.136] M2M_WIFI_RESP_CON_STATE_CHANGED: CONNECTED

[11:45:12.136] winc1500_wifi_cb: 32

[11:45:12.136] M2M_WIFI_REQ_DHCP_CONF: IP is 192.168.0.102

[11:45:12.136] WINC is connected to TP-LINK_258C successfully!

[11:45:12.136] winc1500_wifi_init: Done

[11:45:12.136] Initializing rando.

[11:45:12.136] AWS IoT SDK Version 2.1.1-

[11:45:12.136] Connecting...

[11:45:12.136]   . Seeding the random number generator...

[11:45:12.136]   . Loading the CA root certificate ...

[11:45:12.136]  ok (0 skipped)

[11:45:12.136]   . Loading the client cert. and key...

[11:45:12.447]  ok

[11:45:12.497]   . Connecting to a3hglwcan0yf7v.iot.us-west-2.amazonaws.com/8883

...winc1500_wifi_cb: 20

[11:45:12.547] SERVER IP is 52.88.193.255

[11:45:12.597]  ok

[11:45:12.597]   . Setting up the SSL/TLS structure...

[11:45:12.648] SSL state connect : 0  ok

[11:45:12.648] SSL state connect : 0   . Performing the SSL/TLS handshake...

[11:45:14.256] Verify requested for (Depth 2):

[11:45:14.256] cert. version     : 3

[11:45:14.256] serial number     : 18:DA:D1:9E:26:7D:E8:BB:4A:21:58:CD:CC:6B:3B:

4A

[11:45:14.256] issuer name       : C=US, O=VeriSign, Inc., OU=VeriSign Trust Net

work, OU=(c) 2006 VeriSign, Inc. - For authorized use only, CN=VeriSign Class 3

Public Primary Certification Authority - G5

[11:45:14.256] subject name      : C=US, O=VeriSign, Inc., OU=VeriSign Trust Net

work, OU=(c) 2006 VeriSign, Inc. - For authorized use only, CN=VeriSign Class 3

Public Primary Certification Authority - G5

[11:45:14.256] issued  on        : 2006-11-08 00:00:00

[11:45:14.256] expires on        : 2036-07-16 23:59:59

[11:45:14.256] signed using      : RSA with SHA1

[11:45:14.256] RSA key size      : 2048 bits

[11:45:14.256] basic constraints : CA=true

[11:45:14.256] key usage         : Key Cert Sign, CRL Sign

[11:45:14.256]   This certificate has no flags

[11:45:14.256] Verify requested for (Depth 1):

[11:45:14.256] cert. version     : 3

[11:45:14.256] serial number     : 3F:92:87:BE:9D:1D:A4:A3:7A:9D:F6:28:2E:77:5A:

C4

[11:45:14.256] issuer name       : C=US, O=VeriSign, Inc., OU=VeriSign Trust Net

work, OU=(c) 2006 VeriSign, Inc. - For authorized use only, CN=VeriSign Class 3

Public Primary Certification Authority - G5

[11:45:14.256] subject name      : C=US, O=Symantec Corporation, OU=Symantec Tru

st Network, CN=Symantec Class 3 ECC 256 bit SSL CA - G2

[11:45:14.256] issued  on        : 2015-05-12 00:00:00

[11:45:14.256] expires on        : 2025-05-11 23:59:59

[11:45:14.256] signed using      : RSA with SHA-256

[11:45:14.256] EC key size       : 256 bits

[11:45:14.256] basic constraints : CA=true, max_pathlen=0

[11:45:14.256] subject alt name  :

[11:45:14.256] key usage         : Key Cert Sign, CRL Sign

[11:45:14.308]   This certificate has no flags

[11:45:14.308] Verify requested for (Depth 0):

[11:45:14.308] cert. version     : 3

[11:45:14.308] serial number     : 20:88:52:7E:45:DF:8F:76:68:0D:39:EA:E2:26:12:

D2

[11:45:14.308] issuer name       : C=US, O=Symantec Corporation, OU=Symantec Tru

st Network, CN=Symantec Class 3 ECC 256 bit SSL CA - G2

[11:45:14.308] subject name      : C=US, ST=Washington, L=Seattle, O=Amazon.com,

Inc., CN=*.iot.us-west-2.amazonaws.com

[11:45:14.308] issued  on        : 2017-03-07 00:00:00

[11:45:14.308] expires on        : 2018-03-08 23:59:59

[11:45:14.308] signed using      : ECDSA with SHA256

[11:45:14.308] EC key size       : 256 bits

[11:45:14.308] basic constraints : CA=false

[11:45:14.308] subject alt name  : iot.us-west-2.amazonaws.com, *.iot.us-west-2.

amazonaws.com

[11:45:14.308] key usage         : Digital Signature

[11:45:14.308] ext key usage     : TLS Web Server Authentication, TLS Web Client

Authentication

[11:45:14.308]   This certificate has no flags

[11:45:22.677]  ok

[11:45:22.677]     [ Protocol is TLSv1.2 ]

[11:45:22.677]     [ Ciphersuite is TLS-ECDHE-ECDSA-WITH-AES-256-GCM-SHA384 ]

[11:45:22.677]     [ Record expansion is 29 ]

[11:45:22.677]   . Verifying peer X.509 certificate...

[11:45:22.677]  ok

[11:45:22.796] autoreconnect_set_status...

[11:45:22.796] Subscribing...

[11:45:22.902] aws_iot_mqtt_yield...

[11:45:27.926] ======================================================

[11:45:28.035] Temperature is: 25.50 Celsius

[11:45:28.035] *********************************************************

[11:45:28.035] aws_iot_mqtt_yield...

[11:45:33.123] ======================================================

[11:45:33.276] Temperature is: 26.00 Celsius

[11:45:33.276] *********************************************************

[11:45:33.276] aws_iot_mqtt_yield...

[11:45:38.915] ======================================================

[11:45:39.020] Temperature is: 26.50 Celsius

[11:45:39.020] *********************************************************

## Steps to run the AWS demo

### Setup AWS Account

1.	Create Amazon AWS Account

https://aws.amazon.com/account/

2.	Install and setup Amazon Command Line Interface

http://docs.aws.amazon.com/cli/latest/userguide/cli-chap-getting-set-up.html

3.	Configure AWS Command Line interface

http://docs.aws.amazon.com/cli/latest/userguide/cli-chap-getting-started.html

### AWS Thing and Certificate Creation

1.	Create your IOT Thing

aws iot create-thing --thing-name "yourThingName"

2.	Create certificate and keys

aws iot create-keys-and-certificate --set-as-active --certificate-pem-outfile cert.pem --public-key-outfile publicKey.pem --private-key-outfile privkey.pem

Note the certificate-arn in output. You can also get it by listing all certificates

aws iot list-certificates

### Attach a Policy to the Certificate

1.	Create a simple iotpolicy.json file with following as its contents:

{

"Version": "2012-10-17",

"Statement": [{

"Effect": "Allow",

"Action":["iot:*"],

"Resource": ["*"]

}]

}

This is a simple test policy file allowing all iot actions on all resources

2.	Create the AWS policy using the iotpolicy.json file

aws iot create-policy --policy-name "PubSubToAnyTopic" --policy-document file://iotpolicy.json

3.	Attach the policy to the certificate

Use your certificate-arn in the command

aws iot attach-principal-policy --principal "certificate-arn" --policy-name "PubSubToAnyTopic"

### Build AWS IOT SecureIOT1702 project

- Open the secureIOT1702_led_demo Keil project

- Update subscribe_publish_app.c with your certificates and keys:

  - Root Certificate  :  char rootCA[ ] =  “  “

  - Client Certificate : char clientCRT[ ] =  “ “

  - Client Private Key  : char clientKey[ ] ==  “ “

Example for Root Certificate:

- Update aws_iot_config.h to match your AWS host, client and thing

Example:

- Update CONN_SSID, CONN_PSK_PWD in winc1500_connect.c to match your wifi router.

- Build the Keil project

- Program the firmware binary (targets\ spi_image.bin)  into flash using DediProg SF 100

### AWS Led Demo

- After the firmware is programmed, turn on the secureIOT1702 board

- Once securely connected, the AWS demo will send emulated temperature values to the AWS IoT cloud.

Basically it is posting emulated temperature values to the MQTT topic sdk/test/temperature.

You should be able to see these updating values in AWS IoT Dashboard.

- Open https://aws.amazon.com/iot/

- Sign In to IoT Console

- Go to All Services -> Internet of Things and click on AWS IOT

- On the AWS IoT Console click on “Test” on left hand side pane

- Here we need to subscribe to the following topic:

sdk/test/temperature

The subscribed topic will be added on the left hand side under subscriptions

- Click on the subscribed temperature topic to see the temperature reading updates every 5 seconds.

- To turn on/off the led on the secureIOT1702 board, we need to post to the following  topic

devices/bulb1

In the same “Test” tab in the AWS IoT Console, you can publish to topics. Simply publish ON or OFF to the topic devices/bulb1

-------------------------------------------------- END OF DOCUMENT -----------------------------------------------------------