This is a Beta Release, intended only for evaluation purpose. Contact your local SMSC FAE to get the latest software package.

- Building Customized Image of Windows CE 3.0 Operating System for Intel StrongARM SA-1111 Development Platform

- BUILD-NOTE

- Follow the steps to build a new Windows CE customized image

- Follow the Steps in Intel StrongARM* SA-1110 Development Platform-Windows CE Software-User Guide to build the Default Windows CE Maxall Built for SA11X0BD Platform.

- Build MAXALL built of Windows CE image without any modifications.

- Attaching LAN9000 Drivers to the default built like MAXALL of Windows CE Operating System

Place LAN9000 driver file (Lan9000.dll) in Windows CE Release (_FLATRELEASEDIR) Directory.

LAN9000.DLL is only required file for LAN9000 drivers. This file is present in CE9000.ZIP file.

- FILE MODIFICATIONS

- Edit Setenv.bat:

- set SMSC_LAN9000=1

- Edit PLATFORM.BIB file at the location PLATFORM\<PROJECTPLATFORMNAME>\FILES.

- IF SMSC_LAN9000

- lan9000.dll	    $(_FLATRELEASEDIR)\lan9000.dll              NK  SH

- Follow the instructions in LAN91C96-SA1111DB.doc to add the LAN91C96 software support in Intel StrongARM* SA1111 platform.

- REGISTRY MODIFICATIONS FOR LAN91C96

- Modify the Registry file (Platform.reg) to add the support for LAN91C96. See Below for registry modifications.

- ;Begin LAN91C96 Support

- [HKEY_LOCAL_MACHINE\Comm\LAN9000]

- “DisplayName”=”LAN91C96 Network Adapter”

- “Group”=”NDIS”

- “ImagePath”=”lan9000.dll”

- [HKEY_LOCAL_MACHINE\Comm\LAN9000\Linkage]

- “Route”=multi_sz:”LAN90001”

- [HKEY_LOCAL_MACHINE\Comm\LAN90001]

- “DisplayName”=”LAN91C96 Compatible Network Adapter”

- “Group”=”NDIS”

- “ImagePath”=”lan9000.dll”

- [HKEY_LOCAL_MACHINE\Comm\Ident]

- “Password”=”Password”

- “Domain”=”Domain”

- “UserName”=”UserName”

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms]

- “"BusNumber"=dword:0

- ;; ISA type should be always 1. The chip is in PCMCIA mode but we have to configure it as ISA

- "BusType"=dword:1

- ;; interrupt level. By default the drivers and system is setup to hex 10 but you can always change it. Follow  ;;the instructions in the LAN91C96-SA1111DB.doc to change the Interrupt.

- "Irq"=dword:10

- "NetworkAddress"="00800F000001"  		;;default network address

- "Transceiver"=dword:0 				;;AUTO

- "Duplex"=dword:0ff 				;; duplex: autonegotiate

- "EarlyTransmit"=dword:0 			;;OFF

- "Pcmcia"=dword:0 				;;NO

- "ReceiveTimer"=dword:0

- "Speed"=dword:0ff 				;;Autonegotiate

- [HKEY_LOCAL_MACHINE\Comm\LAN90001\Parms\TcpIp]

- "EnableDHCP"=dword:1 		;;DHCP enabled

- "DefaultGateway"=""

- "UseZeroBroadcast"=dword:0

- "IpAddress"="0.0.0.0"

- "Subnetmask"="0.0.0.0"

- [HKEY_LOCAL_MACHINE\Comm\TcpIp\Linkage]

- “Bind”=multi_sz:”ppp”,”lan90001”

- NOTE: YOU CAN CHOOSE THE ABOVE COMBINATION OF THE MODIFICATIONS IN THE PLATFORM.REG FILE.

- Build CE image:

After doing the required modifications, you can build a new CE image using WinCE.BAT. Follow the instructions provided in Platform Builder for Windows CE 3.0 to build the debug and release versions of the drivers and CE image.