Quick Test.exe (Production Line Device Test Utility)

QuickTest.exe requires the SMSC password filter driver (Smscpswd.inf) to be installed in order to function properly.  This driver is no longer installed by the card reader installer and must be installed manually before running QuickTest.

Installing SMSC password filter driver:

Copy Smscpswd.inf and Smscpswd.sys to your system.  Open the device manager and double click on the USB Mass Storage Device entry in the Universal Serial Bus Controllers section.  In the driver tab, select update driver.   The wizard will assist in installing this driver.  When given the choice, specify to have the wizard display a list of known drivers for this device.  Choose “Have Disk” and browse to where you copied the smscpswd.inf file and select it.  This should give you the option to install “USB Mass Storage Device with Password Protection (WinMe)”.

INI File Requirements:

The ini file should contain the following lines.

- OEMVID = vid

This is the original equipment manufacturer’s VID (Vendor ID) of the device whose descriptor has already been updated. The ‘vid’ is specified as a four digit hexadecimal number.

- OEMPID = pid

This is the original equipment manufacturer’s PID (Product ID) of the device whose descriptor has already been updated. The ‘pid’ is specified as a four digit hexadecimal number.

- INQUIRY_MFG = Inquiry MFG String

This is the string returned by the device as part of the Vendor information in the Inquiry data. This can be of maximum 8 characters.

- INQUIRY_PRODUCT = Inquiry Product String

This is part of the string returned by the device as part of the Product information in the Inquiry data. This can be of maximum 5 characters.

- MAX_DEVICES = n

Specifies the number of devices that are to be tested simultaneously. The ‘n’ is specified as a decimal number and should be >= 1 and <= 4.

- MAX_LUNS = n

Specifies the number of logical units (LUNs) present in the device. The ‘n’ is specified as a decimal number and should be >= 1 and <= 4.

- TEST_SIZE = size

Specifies the amount of data that is to be transferred during read / write tests. The ‘size’ is specified as a decimal number in units of Kilo Bytes (KB). Also, the value for ‘size’ should be >= 64 and <= 5000.

NOTE:

- There can be spaces before and after the ‘=’ (equals) sign, but the total number of characters per line (including spaces) should be LESS THAN (<) 255.

- The ‘vid’, ‘pid’, ‘Inquiry MFG String’ and ‘Inquiry Product String’ are needed only for cleaning the registry after testing. These are not taken into consideration by the application to actually match against the device’s descriptors.

Sample INI File:

Following is a sample ini file for the OEM “AboCom”

OEMVID			= 1276

OEMPID			= 5602

INQUIRY_MFG		= AboCom

INQUIRY_PRODUCT	= UR106

MAX_DEVICES		= 4

MAX_LUNS			= 4

TEST_SIZE			= 1024

Note: The following are valid with respect to the above sample INI file.

- The utility will test 4 devices simultaneously. However, if the actual number of devices connected is less than four, then the utility will test only that number of devices. The remaining number of devices will be listed as “Device not found”. Similarly, if the actual number of devices connected is greater than four, then only the first four devices will be tested.

- The utility will test for 4 LUNs on each device. However, if the actual number of LUNs present in a device is less than four, then the utility will test only that number of LUNs. The remaining number of LUNs will be listed as “Failed” with an error message. Similarly, if the actual number of LUNs present in a device is greater than four, then only the first four LUNs of the device will be tested.

- The tests for writes and reads will use a transfer size of 1024KB.

- After testing, the utility will clean up the registry entries that are associated to values specified in vid, pid, mfg and product.