# API PoE Protocol

## PoE Base Message Structure

PoE messages can be one of the following types: infrastructure, request, and response. Every message is composed of a message header and message body.

### Message Header

Table 1 describes PoE message header parameters.

Table 1: Message Header Parameters

| Header Field | Header Field Length |  | Description |
| --- | --- | --- | --- |
| Code | Unsigned short (16 bits) |  | A unique message code that is set in the message header by host programmer .API infrastructure reserves code 0 for the infrastructure message type. |
| Length |  | Unsigned byte ( 8 bits) | The length of the message (received parameters, not include the header size, not include the “N” ) |

### Message Body

Some messages carry data after the header. API client should use buffer with the length of minimum message length (15 byte). Each command has its own binary code and can be followed by data.

#### GET Messages

All Get messages are from type of empty structures (means no additional data)

The telemetry (reply) carries data after the header

#### SET Messages

Most of Set messages carry data after the header

### Message Structure

typedef struct _POE_BASE_MESSAGE {

unsigned short code; //POE message code

unsigned byte length;//message variable length

char data[1];//place holder of additional data

} POE_MESSAGE, *P_POE_BASE_MESSAGE;

### PoE Primitive interface

- PoE primitive interface messages are based on “PD69200 _BT -PoE Serial Communication Protocol” and are represented as binary commands. Each command has its own binary code and may be followed by data. Empty Primitive uses general structure (see Message Header). for example: Get messages or Set messages that contain only fixed parameters within the 15-byte command.

#### POE API Codes

#define POE_CODE_INIT_POE_API	    API_INFRASTRCTURE_COMMAND_BASE (0)

#define POE_CODE_SET_DOWNLOAD_FILE   API_INFRASTRCTURE_COMMAND_BASE+1

#define POE_CODE_RESET_COMMAND    	        API_SYSTEM_COMMAND_BASE (10)

#define POE_CODE_RES_FAC_DEFAULT		        API_SYSTEM_COMMAND_BASE+1

#define POE_CODE_SAVE_SYS_SETTING		        API_SYSTEM_COMMAND_BASE+2

#define POE_CODE_SET_USER_BYTE_SAVE             API_SYSTEM_COMMAND_BASE+3

#define POE_CODE_SET_PRIVATE_LABEL	        API_SYSTEM_COMMAND_BASE+4

#define POE_CODE_GET_BT_SYSTEM_STATUS           API_SYSTEM_COMMAND_BASE+5

#define POE_CODE_GET_BT_SYSTEM_STATUS_REP       API_SYSTEM_COMMAND_BASE+6

#define POE_CODE_GET_BT_EVENT_CAUSE             API_SYSTEM_COMMAND_BASE+7

#define POE_CODE_GET_BT_EVENT_CAUSE_REP         API_SYSTEM_COMMAND_BASE+8

#define POE_CODE_GET_SYSTEM_STATUS2             API_SYSTEM_COMMAND_BASE+9

#define POE_CODE_GET_SYSTEM_STATUS2_REP         API_SYSTEM_COMMAND_BASE+10

#define POE_CODE_SET_BT_EVENTS_INTERRUPT_MASK   API_SYSTEM_COMMAND_BASE+11

#define POE_CODE_GET_BT_EVENTS_INTERRUPT_MASK   API_SYSTEM_COMMAND_BASE+12

#define POE_CODE_GET_BT_EVENTS_INTERRUPT_MASK_REP API_SYSTEM_COMMAND_BASE+13

#define POE_CODE_SET_INDIVIDUAL_MASK            API_SYSTEM_COMMAND_BASE+14

#define POE_CODE_GET_INDIVIDUAL_MASK            API_SYSTEM_COMMAND_BASE+15

#define POE_CODE_GET_INDIVIDUAL_MASK_REP        API_SYSTEM_COMMAND_BASE+16 #define POE_CODE_SET_BT_POWER_IND_LED           API_SYSTEM_COMMAND_BASE+17

#define POE_CODE_GET_BT_POWER_IND_LED           API_SYSTEM_COMMAND_BASE+18

#define POE_CODE_GET_BT_POWER_IND_LED_REP       API_SYSTEM_COMMAND_BASE+19

#define POE_CODE_SET_BT_CASS_ADD_POWER         API_SYSTEM_COMMAND_BASE+20

#define POE_CODE_GET_BT_CLASS_POWER             API_SYSTEM_COMMAND_BASE+21

#define POE_CODE_GET_BT_CLASS_POWER_REP         API_SYSTEM_COMMAND_BASE+22

#define POE_CODE_GET_SW_VERSION                 API_SYSTEM_COMMAND_BASE+23

#define POE_CODE_GET_SW_VERSION_REP             API_SYSTEM_COMMAND_BASE+24

#define POE_CODE_SET_LOG_SEC_CLEAR_STAMP        API_SYSTEM_COMMAND_BASE+25

#define POE_CODE_GET_BOOT_UP_ERROR_REP          API_SYSTEM_COMMAND_BASE+26

#define POE_CODE_SET_DEV_PARAMETERS         API_DEVICE_COMMAND_BASE (40)

#define POE_CODE_GET_BT_DEV_STATUS          API_DEVICE_COMMAND_BASE+1

#define POE_CODE_GET_BT_DEV_STATUS_REP      API_DEVICE_COMMAND_BASE+2

#define POE_CODE_SET_DEV_REGISTER           API_DEVICE_COMMAND_BASE+3

#define POE_CODE_GET_DEV_REGISTER           API_DEVICE_COMMAND_BASE+4

#define POE_CODE_GET_DEV_REGISTER_REP       API_DEVICE_COMMAND_BASE+5

#define POE_CODE_GET_DEV_ERROR_LOG          API_DEVICE_COMMAND_BASE+6

#define POE_CODE_GET_DEV_ERROR_LOG_REP      API_DEVICE_COMMAND_BASE+7

#define POE_CODE_GET_LOG_SEC_STATUS         API_DEVICE_COMMAND_BASE+8

#define POE_CODE_GET_LOG_SEC_STATUS_REP     API_DEVICE_COMMAND_BASE+9

#define POE_CODE_SET_TEMP_MATRIX               API_PORT_COMMAND_BASE (50)

#define POE_CODE_GET_PHY_PORT_TMP_MATRIX       API_PORT_COMMAND_BASE+1

#define POE_CODE_GET_PHY_PORT_TMP_MATRIX_REP   API_PORT_COMMAND_BASE+2

#define POE_CODE_PROGRAM_GLOBAL_MATRIX         API_PORT_COMMAND_BASE+3

#define POE_CODE_GET_PHY_PORT_ACT_MATRIX       API_PORT_COMMAND_BASE+4

#define POE_CODE_GET_PHY_PORT_ACT_MATRIX_REP   API_PORT_COMMAND_BASE+5

#define POE_CODE_GET_PORTS_DEL_POWER_STATE     API_PORT_COMMAND_BASE+6

#define POE_CODE_GET_PORTS_DEL_POWER_STATE_REP API_PORT_COMMAND_BASE+7

#define POE_CODE_SET_BT_PORT_PARAMETERS        API_PORT_COMMAND_BASE+8

#define POE_CODE_GET_BT_PORT_PARAMETERS        API_PORT_COMMAND_BASE+9

#define POE_CODE_GET_BT_PORT_PARAMETERS_REP    API_PORT_COMMAND_BASE+10

#define POE_CODE_GET_BT_PORT_STATUS            API_PORT_COMMAND_BASE+11

#define POE_CODE_GET_BT_PORT_STATUS_REP        API_PORT_COMMAND_BASE+12

#define POE_CODE_GET_BT_PORT_COUNTERS          API_PORT_COMMAND_BASE+13

#define POE_CODE_GET_BT_PORT_COUNTERS_REP      API_PORT_COMMAND_BASE+14

#define POE_CODE_GET_BT_PORT_CLASS             API_PORT_COMMAND_BASE+15

#define POE_CODE_GET_BT_PORT_CLASS_REP        API_PORT_COMMAND_BASE+16

#define POE_CODE_SET_BT_PORT_LAYER2_LLDP_PD_REQUEST       API_PORT_COMMAND_BASE+17

#define POE_CODE_GET_BT_PORT_LAYER2_LLDP_PSE_DATE         API_PORT_COMMAND_BASE+18

#define POE_CODE_GET_BT_PORT_LAYER2_LLDP_PSE_DATE_REP     API_PORT_COMMAND_BASE+19

#define POE_CODE_GET_BT_PORT_LAYER2_LLDP_PD_REQUEST       API_PORT_COMMAND_BASE+20

#define POE_CODE_GET_BT_PORT_LAYER2_LLDP_PD_REQUEST_REP   API_PORT_COMMAND_BASE+21

#define POE_CODE_SET_PORT_LAYER2_CDP_POWER_UP_ALTB        API_PORT_COMMAND_BASE+22

#define POE_CODE_GET_PORT_LAYER2_CDP_POWER_UP_ALTB        API_PORT_COMMAND_BASE+23

#define POE_CODE_GET_PORT_LAYER2_CDP_POWER_UP_ALTB_REP    API_PORT_COMMAND_BASE+24

#define POE_CODE_SET_PORT_LAYER2_CDP_PD_REQUEST           API_PORT_COMMAND_BASE+25

#define POE_CODE_GET_BT_PORT_LAYER2_CDP_PSE_DATA          API_PORT_COMMAND_BASE+26

#define POE_CODE_GET_BT_PORT_LAYER2_CDP_PSE_DATA_REP      API_PORT_COMMAND_BASE+27

#define POE_CODE_GET_TOTAL_POWER            API_POWER_MNGT_COMMAND_BASE (80)

#define POE_CODE_GET_TOTAL_POWER_REP        API_POWER_MNGT_COMMAND_BASE+1

#define POE_CODE_SET_POWER_BANKS            API_POWER_MNGT_COMMAND_BASE+2

#define POE_CODE_GET_POWER_BANKS            API_POWER_MNGT_COMMAND_BASE+3

#define POE_CODE_GET_POWER_BANKS_REP        API_POWER_MNGT_COMMAND_BASE+4

#define POE_CODE_GET_POWER_SUPP_PARAMS      API_POWER_MNGT_COMMAND_BASE+5

#define POE_CODE_GET_POWER_SUPP_PARAMS_REP  API_POWER_MNGT_COMMAND_BASE+6

#define POE_CODE_GET_BT_PORT_MEAS           API_POWER_MNGT_COMMAND_BASE+7

#define POE_CODE_GET_BT_PORT_MEAS_REP       API_POWER_MNGT_COMMAND_BASE+8

#define POE_CODE_SET_DERATING_DATA          API_POWER_DER_COMMAND_BASE (90)

#define POE_CODE_GET_DERATING_DATA          API_POWER_DER_COMMAND_BASE+1

#define POE_CODE_GET_DERATING_DATA_REP      API_POWER_DER_COMMAND_BASE+2

#define POE_CODE_SET_DERATING_USER_TEMP     API_POWER_DER_COMMAND_BASE+3

#define POE_CODE_GET_DERATING_USER_TEMP     API_POWER_DER_COMMAND_BASE+4

#define POE_CODE_GET_DERATING_USER_TEMP_REP API_POWER_DER_COMMAND_BASE+5

#define POE_CODE_GET_DERATING_SYS_MEAS      API_POWER_DER_COMMAND_BASE+6

#define POE_CODE_GET_DERATING_SYS_MEAS_REP  API_POWER_DER_COMMAND_BASE+7

#define POE_CODE_DOWNLOAD_COMMAND           API_SOFTWARE_DOWNLOAD (100)

#### POE API data

API Data is composed of structure for any command/request/telemetry/response from host to PoE controller and vice versa.

The structure represents the variant component in any such an element above. For each Request (POE_CODE_GET_XXX) the structures are empty.

Bellow examples of interface structures:

##### Set Interrupt Mask

typedef struct _POE_SET _INTERRUPT _MASK     {                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                              	UINT16	 maskRegister;			/* See Comm. Protocol */

} POE_ SET _INTERRUPT _MASK, *P_ POE_ SET _INTERRUPT _MASK;

##### Get Interrupt Mask Telemetry

typedef struct _POE_GET _INTERRUPT _MASK_REP     {                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                                              	UINT16	 maskRegister;			/* See Comm. Protocol */

} POE_ GET _INTERRUPT _MASK_REP, *P_ POE_ GET _INTERRUPT _MASK_REP;

##### Get BT System Status

typedef struct _POE_GET _BT_SYS _STATUS_REP     {

BOOL bootUpErr;

UINT8 cpuStatus2;

UINT8 factoryDefault;

UINT8 ramPrivateLabel;

UINT8 nvmUser;

UINT8 foundDevices;

UINT16	 interruptRegister;			/* See Comm. Protocol */

} POE_ GET _ GET_BT_SYS _STATUS _REP, *P_ POE_ GET_BT_SYS _STATUS _REP;

## PoE Interface

The following services (functions)are interface for client to use for Host PoE Exchange

### PoE_RC_e POEAPOpenSession(IN const char *devName,              PoE_Comm_Dev_e devType, UINT16 devCommDelay ,UINT8 *params, IN unsigned int apiVersion);

Initialize hardware communication between host and PoE. Done using Hardware Abstraction Layer method.

Parameters:

DevName - Device name (path on linux)

DevType – one of hardware devices used by client (‏2‏2)

devCommDelay – delay before command answer

params-  pointer to attribute for specific hardware device

apiVersion – the current API version that programmer wants to bind to.

Return:

Reply status enumeration(Appendix)

### PoE_RC_e POEAPCloseSession(void)

Close Host PoE transaction session

Parameters:

None

Return:

Reply status enumeration(Appendix)

### PoE_RC_e POEAPITranslateMessage(IN_OUT P_POE_BASE_MESSAGE pmsg);

Translate specific message code and get the reply

Parameters:

Pmsg - Pointer to send and reply message

Return:

Reply status enumeration (Appendix)

Bellow examples (for host programmer) of implementation for set and get messages:

#### POE_CODE_SET_INTERRUPT_MASK

P_POE_BASE_MESSAGE pbmsg = (P_POE_BASE_MESSAGE) buffer;

P_POE_SET_INTERRUPT_MASK pint_mask;

PoE_RC_e rc;

pbmsg->code = POE_CODE_SET_INTERRUPT_MASK;

pmsg->length =sizeof(POE_ SET _INTERRUPT _MASK);

pint_mask = (P_ POE_SET_INTERRUPT_MASK)pbmsg->data;

pint_mask ->maskRegister = 0xb011;

rc =  POEAPITranslateMessage(pbmsg);

#### POE_CODE_GET_INTERRUPT_MASK

P_POE_BASE_MESSAGE pbmsg = (P_POE_BASE_MESSAGE) buffer;

P_POE_GET_INTERRUPT_MASK_REP pint_mask;

PoE_RC_e rc;

UINT16 MaskRegister_rep;

pbmsg->code = POE_CODE_GET_INTERRUPT_MASK;

pmsg->length =0;

rc =  POEAPITranslateMessage(pbmsg);

// reply

If((rc == PoE_RC_SUCSESS) &&(MtilReqBuffer->code == POE_CODE_GET_INTERRUPT_MASK_REP)

{

pint_mask = (P_ POE_GET_INTERRUPT_MASK_REP)pbmsg->data;

maskRegister_rep = pint_mask ->maskRegister;

}

### POE_FLOWCHART_SUCCESS getFlow(P_POE_GET_FLOWCHART pGetFlowchart)

Get the flowchart

Parameters:

pGetFlowchart - Pointer to a structure of POE_GET_FLOWCHART. This pointer is changed in the function.

Return:

Reply : 0 – success.

Bellow example (for host programmer ) of implementation get flowchart:

getFlowchart = (P_POE_GET_FLOWCHART)malloc(sizeof(POE_GET_FLOWCHART));

POE_FLOWCHART_SUCCESS res = getFlow(getFlowchart);

// get the relevant information from getFlowchart

free(getFlowchart);

The structure:

typedef struct _POE_GET_FLOWCHART {

POE_SYSTEM_OK_REG_VMAIN  systemOKRegVmain;

POE_SYS_OK_REG_PWR_PREC  systemOKRegPwrPrec;

POE_SYS_OK_REG_PWR_VAL   systemOKRegPwrVal;

BOOL portsStatusListBool[48]; // to know if there is an event in the port

BOOL portsClassListBool[48]; // to know if we need to call get BT port class

BOOL portsCountersListBool[48]; // to know if we need to call get BT port counters

BOOL deviceListBool[12]; // to know if there is an event in the device

POE_GET_BT_DEV_STATUS_REP deviceList[12];

POE_GET_BT_PORT_STATUS_REP portsStatusList[48];

POE_GET_BT_PORT_CLASS_REP portsClassList[48];

POE_GET_BT_PORT_COUNTERS_REP portsCountersList[48];

POE_GET_BT_PORT_STATUS_REP nePortsStatusList[48];

// Get Total Power

UINT16 powerConsumption;

UINT16  calcPower;

UINT16 availablePower;

UINT16  powerLim;

UINT8  powerBank;

UINT16  vmainVoltage;

}POE_GET_FLOWCHART, *P_POE_GET_FLOWCHART;

## Host PoE software Low Level Design Concept

### Host PoE SW Infrastructure

The Host PoE software is the infrastructure module that receives all Host client packets from one side and issues all translations to the PoE Controller. It interfaces with the API library and implements the logic and performs the translations to and from PoE controller. It is executed as a single thread, and all interfaces are synchronous and blocking.

### Host PoE SW API

The Host PoE SW API is the library responsible for processing the binary codes (of commands , requests and telemetries ) and for translating between Host and PoE controller. This API interfaces with the SW infrastructure from one side and the client application from other side.

Flow Chart for success Host-PoE Communication services (routines) describes success session between the API and infrastructure

### Host PoE framework

The Host PoE SW maintains a Data Base to manage Commands and requests internal information. The DB is accessed by a unique message code primitive of the command /request and includes the fixed parameters of the appropriate 15- bytes protocol. since request and telemetry are identified with consequence codes, they are easily handled by retrieving appropriate 15-byte fixed parameters.

### Design Advantages

#### Easy for client to implement

Host programmer is concerned of only relevant data need to send/get to/from PoE controller

#### Modular design

Exchanging the commands/requests is similar for all communication Protocol commands/requests up to different parameters

#### Robustness design

Updates within communication protocol have minor effects on Client SW implementation

#### Hardware Is Abstracted

Host programmer defines the communication device within dedicated interface function

#### 15- byte Protocol coupling

Each command request has unique code for different15 -byte protocol data

## Delivered software structure

### Software package structure

The zip code contains two folders:

Host_poe_api

Host_poe_hal

### Host PoE API

This folder contains the up level of the API.

#### Host PoE API

The main interface file for host programmer is host_poe_api/inc/host_poe_api.h

This file contains the POE 15-byte commands structures and functions for programmer to use.

The file host_poe_api/src /host_poe_api.c the implementation of the API.

#### Host PoE Client

This file (host_poe_api/src /host_poe_client.c) is External Linux thread contains all Comm protocol commands (host_poe_client.c). This thread was created and tested on Linux Lubuntu Core distribution

### Host PoE HAL

This folder is the Hardware Abstraction Layer and contains all the low level (communication device and Operating System) required by Host PoE API.

#### Host PoE Comm Device

This file (host_poe_api/src /host_poe_comm_device.c) executes the communication specific device (currently UART or I2C). The host programmer selects the specific device while opening the channel through POEAPIOpenSession. Once selecting the device, it will be used until POEAPICloseSession is being called

#### Host PoE Comm OS

This file (host_poe_api/src /host_poe_comm_os.c) provides with interface to specific Host Programmer Operation system. On entire package LINUX OS is used. The programmer may fill the OS function with any proper OS interface.

### API Version

Each API revision increases the version number HOST_POE_API_VER which is defined within host_poe_api/inc/host_poe_api.h

The interface POEAPOpenSession refers the number of API revision that host programmer wish to use. In case the version is different than what is actually used – error will return

### Compile the sources

All The software code inside this folder is ANSI-C compliant.Makefile of the package is optimized for GNU GCC.

## Flow Chart for success Host-PoE Communication

PoE IC’s

CLIENT

Host PoE Link

POE API

15 byte protocol reply

15 byte protocol Request

Build 15 Byte message of  XXX  Request

POE_CODE_GET_XXX

POEAPITranslateMesage()

Build 15 Byte message of  XXX  Reply

POE_CODE_GET_XXX_REPLY

POE_RC_SUCSESS

POE_RC_SUCSESS

15 byte protocol command

Build 15 Byte message of  XXX Command

POE_CODE_SET_XXX

POEAPITranslateMesage()

POE_RC_SUCSESS

POE_RC_SUCSESS

Initialize Data Base

Initialize OS resources

POE_CODE_INIT_API

POEAPITranslateMesage()

POEAPIInit()

# Appendix

Typedef enum{

PoE_RC_SUCSESS,

PoE_RC_ERR_RESET_DETECT,

PoE_RC_ERR_COMM_DEVICE,

PoE_RC_ERR_COMM_TIMEOUT,

PoE_RC_ERR_WRONG_CS,

PoE_RC_ERR_SUBJECT_BYTES,

PoE_RC_ERR_DATA_BYTES,

PoE_RC_ERR_KEY_VALUE,

PoE_RC_ERR_OUT_OF_RANGE,

PoE_RC_ERR_ECHO_BYTE,

PoE_RC_ERR_MSG_TYPE,

PoE_RC_ERR_MUTEXE_INIT,

PoE_RC_ERR_MUTEX_LOCK_UNLOCK,

PoE_RC_ERR_SLEEP_FUNC,

PoE_RC_ERR_INIT,

PoE_RC_ERR_DRIVER,

PoE_RC_ERR_COMM_DEVICE_NAME

}PoE_RC_e;

Typedef enum{

PoE_Comm_Dev_UART,

PoE_Comm_Dev_I2C,

PoE_Comm_Dev_SUB20,

PoE_Comm_Dev_AARDVARK,

PoE_Comm_Dev_SharedMeM,

}PoE_Comm_Dev_e;