Commands to be used:

4.1.7	Get BT Event Cause

4.2.2	Get BT PoE Device Status

4.3.8	Get BT Port Status

4.3.9	Get BT Port Counters

4.3.11	Get BT Port Class
4.4.5	Get BT Port Measurement

4.4.1	Get Total Power

Recommended Flow:

On Going Flow

Do While system_state = on going

Use   4.1.7 Get BT Event Cause

Check for Ports event (P list), device events(D list), system event.

Create a list of ports with no event (NE list). 'P list + NE list = All system ports list.

Check SystemOKReg and report on Vmain out of range or power budget limitation.

'  Scan the devices that caused an event using (D list)

IF device event then

For D=Device(From D list) to D list End

Use   4.2.2 Get BT PoE Device Status (D)

Host update its own information with the relevant information that is being received by the telemetry of 4.2.2  request.

The Host is recommended to use the following information ==>  Device Event, Device Status, Measured temperature, Reset/POR Counter.

Next D

End IF

' Scan the ports with the Event: Range to be viewed (P list)

For P=Port(From P list) to P list End

Use  4.3.8 Get BT Port Status with Port (P)

Host update its own information with the relevant information that is being received by the telemetry of 4.3.8  request.

The Host is recommended to use the following information ==>  Port Status, Port Event, Last Error Status, Assigned Class, Measured Port Power.

IF (port status = Any BT delivering power) and (Port Event bit 0 = 1)  then  'Port turned on event check

Use   4.3.11 Get BT Port Class (P)

Host updates its own information with the relevant information that is being received by the telemetry of 4.3.11 request.

End IF

IF (port status = any error) and (Port Event bit 2 = 1) then  'Counters Related Event check

Use  4.3.9  Get BT Port Counters (P) and add the error counters value to the relevant error counters in the Host (MIB counters or similar).

Note: "Last Error Status" field can be used to generate MIB counters instead of 4.3.9

End IF

Next P

'  Scan all the remaining ports that were not scanned in the P list by using NE list

For NE=Port(From NE list) to NE list End

Use       4.4.5 Get BT Port Measurement with Port (NE) 4.4.5	Get BT Port Measurement

Host updates its own information with the relevant information that is being received by the telemetry of 4.4.5 request. The Host is recommended to use the following information ==> Measured Port Power.
Next NE

' Start System update

Use   4.4.1 Get Total Power

Update the Host with the relevant information that is being received by the telemetry of 4.4.1 request.

Imain = (Power Consumption Field * 100 / Vmain Voltage Field)[mA]  'Host calculate system current by using the following equation

Loop

Timing calculations for UART:

A command transaction is structured from : TX Message + PoE Comm Task execution + RX Message = (150bit / 19200 )*2 + 11mSec = 7.8125mSec * 2 + 11mSec = 0.026625 Sec

Typical system in steady state:

Number of  messages = 1 + 48 + 1 = 50
50 * 0.026625 = 1.33 sec

Worst case calculations:

Number of  messages = 1 + 12 + (48 * 2) + 1 = 110
110 * 0.026625 = 2.93 sec

Timing calculations for I2C 400KHz:

Typical:  50 * 12mSec = 0.6sec
Worst case: 110 * 12mSec = 1.32sec

Note: The calculations assume that the host is polling the message ready signal.

Conclusions: 
1. To enable 2 messages Pipeline in UART.
2. To verify that all punishments time outs that are not line detection fail, are 5sec and up. (Including UDL due to 4Pair PDs)