# SMSCEND_README.doc: SMSC LAN91C100FD Enhanced Network Driver

This file contains specific information for the SMSC LAN91C100FD Full Duplex 10/100 Mbps Ethernet Controller Enhanced Network Driver (END).  Specifically, this file contains information concerning installation, caveats, errata and specific documentation that the user must be aware of before using this END.

Additionally, the target board's manual README and target.(txt/nr) file provides board-specific information necessary to run VxWorks, and should be read before this END is used.

________________________________________________________________________

RELEASE 1.000/0 , END Driver Initial Release

Initial release of the END for the SMSC LAN91C100FD board.

1.0 Required Documentation

The following documents should be in your possession to understand END technology:

* SENS for Tornado Component Release Supplement 1.0 (Edition 1)

* SENS for Tornado Release Notes 1.0 (Edition 1)

2.0 Deliverables

The files that are delivered with the SMSC LAN91C100FD END are as follows:

* "smscend.obj" - SMSC LAN91C100FD END object

* "SMSCEND_README.doc" - README file for SMSC LAN91C100FD END (this file – Microsoft Word format)

* "configNet.h" -   END configuration file for the FSMC END

* “sysLib.c or Makefile” – Method to include END into vxWorks

3.0 "configNet.h"

The user must set up an end Tbl that tells VxWorks to use the new END drivers.  This table is defined in target/config/<bspname>/configNet.h.  For example, if the user wanted to create two (2) ENDs, one that supported and one that did not, the user would first edit configNet.h to include the statements:

#define FSMC_LOAD_FUNC   fsmcEndLoad

/*

* The initialization string format is:

*

* unit:ivec:ilevel:ioAddr:duplex:PHYAddr:speed

* Note: ivec = (INT_NUM_IRQ0 + INT_VEC_FSMC)

*       ivec = ( 0x20 + 0x0b) = 0x2b

*/

#define FSMC_LOAD_STRING "0x2b:0x0b:0x340:0x0:0x0:0x0:-1"

IMPORT END_OBJ* FSMC_LOAD_FUNC (char*, void*);

The user must also edit the definition of the endTbl to include the following:

END_TBL_ENTRY endDevTbl [] =

{

{ 0, FSMC_LOAD_FUNC, FSMC_LOAD_STRING, 0, NULL, FALSE},

{ 0, END_TBL_END, NULL, 0, NULL, FALSE},

};

When using ENDs, the user can define as many network devices (one per line) as exist in their system.  All of the devices in the endTbl are loaded and started at boot time.

If the user wants to use multiple END devices, the user must define IP_MAX_UNITS to be the maximum number of units to be used.  The default is one (1).  If ipattach () is called for a unit that is greater than IP_MAX_UNITS, an error is returned.

4.0 Changes to "sysLib.h"

There are two (2) ways to include the driver into VxWorks. If one has the source, one can modify the sysLib.c file in the BSP to append the line: #include “smscend.c.”  During compile time the code will be rebuilt and linked into the vxWorks image.

5.0 Changes to "Makefile"

Add the "smscend.obj" name to the MAC_EXTRA line in the "Makefile" for the BSP. Please do not use the quotation marks.

6.0 Errata

* The SENS for Tornado Release Notes and Component Release Supplement both reference the incorrect location for the "configNet.h" file.  It is intended to reside in the BSP directory.