SMSC 91C100 Driver Installation Notes

Driver Version 1.000/0

- Purpose

The purpose of this document is to describe the process in which to include the SMSC LAN91C100 BSD4.3/4.4 drivers into the Wind River Systems (WRS) vxWorks kernel.

- Documentation

For additional information, please refer to the following WRS Manuals:

- Tornado User’s Guide

- Tornado Programmer’s Manual

- Overview

There are essentially two (2) BSD Netif drivers provided. The word essentially is used because the code is the same but there are conditional compiles as a result of options in the config.h file. They are:

- BSD4.3 Netif Driver for Tornado 1.0.1

- BSD4.4 Netif Driver for Tornado 1.0.1 with SENS option

Each of these drivers will be discussed in the following sections.  This document assumes that the reader understands the basics of scaling and configuring vxWorks.

- BSD4.3 and BSD4.4 Netif Drivers

Tornado is shipped with what has come to be termed BSD4.3 support.  These types of drivers differ insignificantly from BSD4.4 code but vastly from ENDs.  It is the intention of WRS to have the company and Customers use ENDs exclusively in the future.

There are some considerations to keep in mind when attempting to configure a BSD4.3 or BSD4.4 Netif driver into your system.  The major consideration is whether one has source code or not.

If you have source code, there are two files that need to be enhanced. They are sysLib.c and config.h.  These files are on the distribution disk.  SysLib.c must be modified to “#include” the driver source code.  Config.h needs to be updated to contain the “fsmc” parameters.  Please refer to the files for the necessary and proper changes.

If you do not have source code, there are two (2) files that need to be modified as well.  Config.h needs to be modified to contain the “fsmc” parameters.  In addition, the  Makefile “MACH_EXTRA” line needs to be changed to include the driver object.  Please note that you should not name the code with a “.o” extension.  It will be deleted as a result of a “make clean” procedure.  It is recommended to use an extension such as “.obj!”

The changes to config.h are as follows:

/* Switch Bank determine RJ45 (Twisted Pair) or MII Connection */

#ifdef  INCLUDE_FSMC

#undef BSD43_DRIVER                 /* Need to conditionally compile code */

#define IO_ADRS_FSMC    0x340

#define INT_LVL_FSMC    0x0b

#define INT_VEC_FSMC    (INT_NUM_IRQ0 + INT_LVL_FSMC)

#define CONFIG_FSMC     1         /* 0=RJ45 or 1=MII */

#define SPEED_FSMC      0       /* No Auto-Negotiation Support in this  */

/* release so set desired SPEED;             */

/* 0=10BaseT; 1=100BaseT                     */

#define PHYAddr_FSMC    0    /* MAC can control up to 32 PHYs           */

/* Support for one in this release (Def=0) */

#define RX_MODE_FSMC    1       /* 0=interrupt level; 1=task level  */

#define DUPLEX_FSMC     0      /* 0=Half Duplex; 1=Full Duplex  */

#endif  /* INCLUDE_FSMC */

The major difference between BSD4.3 and BSD4.4 is the line BSD43_DRIVER.  If you are using the code as a BSD4.3 entity, “#define” the line, otherwise “#undef “the line.

The change to sysLib.c is to add the following line:

#include "if_fsmc.c"

If you do not have source, make the following type of change to the Makefile:

MACH_EXTRA     =   driver_name.obj

- Benchmarks

Benchmark numbers were executed with typical Blaster/Blastee code (code is provided).  These numbers demonstrate limitations more of the ISA bus, internal architecture of the chip and the Windows ’95 operating system (and drivers of said network interface in that system) rather than maximum throughput through the vxWorks stack.

- Caveats and Errata

No known issues at this time!