# Data: 7-12-99								NTTrouble4.00.doc

# Subject:  Trouble Installation of Hampshire NT 4.00 Drivers

Problem:

- Using older TSHARC serial firmware

Description:

The TSHARC serial driver loads but fails to work after the system is restarted.  This problem usually occurs because the TSHARC serial firmware is older causing the NT operating system to detect it as a Logitech compatible serial mouse.  In this case the driver will fail quietly and may enter a comment in the event-viewer indicating that the TSHARC controller has failed to load due to an I/O conflict.

Solution:

To resolve this problem modify the boot.ini file to force NT Detect to not try detecting a mouse on the serial port that the TSHARC controller is connected to.  Use the following procedure to modify the boot.ini:

- Start Windows Explorer

- Double Click on the ROOT directory (C:)

- Find the file “boot.ini”

- Right Click on “boot.ini”

- Select Properties

- Turn off the Read-only Attribute

- Select OK

- Double Click on “boot.ini”, This should open the file in Note Pad

- “boot.ini” should look something like this:

[boot loader]

timeout=30

default=multi(0)disk(0)rdisk(0)partition(1)\WINNT

[operating systems]

multi(0)disk(0)rdisk(0)partition(1)\WINNT="Windows NT Workstation Version 4.00"

multi(0)disk(0)rdisk(0)partition(1)\WINNT="Windows NT Workstation Version 4.00 [VGA mode]" /basevideo /sos

- Under “[operating systems] find the selection you normally use to boot Windows NT

- Add the following to this line:

/NOSERIALMICE=COMx

Where x is the Comport that the touch screen controller is connected to.  For example if the touch screen is connected to COM1 add the following:

/NOSERIALMICE=COM1

If you are using multiple touch screen monitors use the following:

/NOSERIALMICE=COMx,y,z,…

Where x, y, and z are the Comport number.

For example to modify the “boot.ini” shown above for a touch screen on COM1 modify the first line under [operating systems]:

[boot loader]

timeout=30

default=multi(0)disk(0)rdisk(0)partition(1)\WINNT

[operating systems]

multi(0)disk(0)rdisk(0)partition(1)\WINNT="Windows NT Workstation Version 4.00" /NOSERIALMICE=COM1

multi(0)disk(0)rdisk(0)partition(1)\WINNT="Windows NT Workstation Version 4.00 [VGA mode]" /basevideo /sos

- Save “boot.ini”

- In Windows Explorer Right Click on “boot..ini”

- Select Properties

- Select Read-only under Attributes

- Select OK

- You can now Restart your computer and the touch screen will work.

Problem:

- After installing the TSHARC NT driver the standard mouse no longer functions

Description:

After installing the Hampshire TSHARC Windows NT driver the standard mouse no longer functions.  This problem has two possible solutions:

Solution 1:

Go to the Mouse control panel and reinstall the mouse driver.

Solution 2:

Use the Registry editor and verify/set the Start flag.  Use the following procedure:

- From the Start Menu select RUN

- Type Regedit and select OK

- When the Registry Editor starts Select HKEY_LOCAL_MACHINE

- Select SYSTEM

- Select CurrentControlSet

- Select Services

- Select Sermouse

- On the right side of the Registry editor the value keys will be listed verify that Start is set to 1

- If Start is not set to 1 double click on the Start key and change it to 1

- Exit the Registry editor.  You do not have to save the changes this will be done automatically

- Restart the computer

Problem:

- How to install multiple TSHARC controllers:

Description:

Installing multiple TSHARC controllers

Solution:

To install multiple TSHARC controllers just repeat the installation procedure described in the TSHARC user manual.  The THSHARC calibration program will automatically detect that multiple TSHARC controllers are on the system and will ask the user which TSHARC controller to calibrate.